EM36 Active throttle and its relationship with Engine Load

- Option Act T Demand Source/In selects an input for parameter Act T Demand raw.

- Parameter Act T Demand raw is constrained by options Act T Demand min and Act T Demand max.

- If out of demand Min-Max range, option Act T Demand Default used instead.

- If option No Idle Control is set to ‘Off’ then:-

Any value in parameter Act T Idle Target is added from idle speed controller using option   	Act T Idle M.

- If option Act T Speed Input is activated by switch:-

Then a correction from the speed controller parameter Act T Speed Correct, which is 	derived from the [Act T Speed Correct] table, is added. This adjustment is specified as a 	percentage.

- When either option Throttle from Act T Demand or Act Throttle = ‘On’, then:-

Parameter TPS raw = Act T Demand.

- The parameter Throttle is derived from (TPS raw – TPS min) / (TPS max – TPS min) x 100.

- When options MAP for Load and MAF for Load = ‘Off’, then Engine Load = Throttle.

- Note that parameter Engine Load has a user scalar Load Scalar and Load Offset useful if load is defined from a pressure of flow.