	[image: image1.png]



	General Engine Management Systems Ltd.

	
	Doc No: 103187
	103187 DA3 Quick Start Guide Rev3.doc



DA3 QUICK START GUIDE
Please follow these steps before using your DA3.
Wire the DA3 mating connectors into the vehicle as follows:

	Pin
	Connector 1
	Connector 2

	1
	12v Power
	Sensor 5v

	2
	Power Ground
	Analogue 1

	3
	Beacon
	Analogue 2

	4
	RS232
	Analogue 3

	5
	CAN2 Hi
	Analogue 4

	6
	CAN2 Lo
	Analogue 5

	7
	CAN1 Hi
	Analogue 6

	8
	CAN1 Lo
	Analogue 7

	9
	Analogue 8
	Sensor Ground


The mating connectors are as follows:


Connector 1 part number: ASDD606-09-SN.


Connector 2 part number: ASDD606-09-PN.

The beacon input should be momentarily switched to ground to indicate a beacon trigger. If the beacon is not being triggered or is not used, the pin should be left unconnected.

You should install the latest release of GEMS Data Analysis (GDA) from http://www.gems.co.uk/?content=pages&id=software-downloads. There are no drivers to install.

To format the USB Flash drive, firstly, insert DA3 USB Flash Drive into PC.
(If you are re-formatting the drive, it is probably worthwhile taking a copy of the DA3_INTO.TXT file before reformatting)
Format as FAT32 with 4096 bytes allocation unit.

On the reformatted USB Flash drive, create/restore a text file called DA3_INFO.TXT.

Unplug from the PC and plug into the DA3 with power off, then power the unit on, wait for a few seconds, then unplug from DA3 and plug back into PC.

Flash Drive should appear with a Volume Label of “DA3 No Firm”.

Open the DA3_INFO.TXT file and check that the Serial Number is correct.

You can create or edit a DA3 setup in GDA by going to Logger > New Logger Setup (to create a new logging setup file) or Logger > Load Logger Setup (to load a predefined logging setup) in the GDA software. Plug in the DA3 memory stick via the USB to memory stick adaptor to upload a setup. Once a setup has been successfully loaded onto the memory stick, plug the DA3 memory stick back into the DA3 and it is ready to start logging.

Details on how to create a new logging setup file, can be found in either the GDA (Pro) manual on our website, or via the extensive ‘Help’ feature in the software itself, in the Setup Editor section. 

You can download the log by clicking on the message toaster in the bottom right of the screen if GDA is the active window when the DA3 USB Flash stick is inserted into the PC, or by going to Logger > Download Log.

The status of the DA3 can be found by looking at the two LEDs on the front of the case. Refer to the following table for their meaning:

	Log LED
	Power LED

	
	Green
	Blue
	Cyan
	Red
	Red Flash

	Green
	Logging
	
	
	
	

	Green Flash
	Logging, 5% remaining
	
	
	
	

	Blue
	
	
	
	
	Corrupt or missing setup

	Blue Flash
	No Serial data
	
	
	
	

	Cyan
	Log Full
	
	Boot mode – Firmware Programming
	
	

	Red
	
	
	
	
	USB Flash fail

	Red Flash
	No CAN data
	
	
	
	

	Off
	Main app – USB connected
	Main app – No USB connected
	Boot mode
	Application invalid – reprogram required
	Unspecified error in CAN or Serial setup 


[image: image1.png]With the LEDs below the memory stick slot, the Power LED is to the left and the Log LED is to the right:

[image: image2.png]








   Power LED   Memory   Log LED

103187 DA3 Quick Start Guide Rev3.doc
2 of 2
2 June 2017


