EM/GDi/RX 80 – Direct Injection

Support for Bosch HPI5 driver for Bosch HDEV5 injectors, much of the MSV control is programmable. See HDP5 options that have aliased parameters that can be sent out on CAN-bus to configure HPI5.

In the Bosch HPI5 MSV High Pressure Pump, is controlled with 2 timed signals.

Set:

HPP Controller = On, PWM15&16 VANOS = On, PWM15&16 MSV = On, PWM15 and PWM16 = On. The ON signal is generated with PWM16 SEL0 with PWM15.

The peak or “pull-in” time is given in ms by [HPP Peak] table versus battery voltage.

The hold time is given in teeth by [HPP Hold] table versus engine speed.

For best resolution with a 60-2 crank pattern, there are 20 teeth per rev, so for 40deg 2.2222teeth.

The start angle is given with the base [HPP Pressure Duty] map, corrected with a feedback loop controlling to [HPP Pressure Target] map, and proportional and integral terms from [HPP Pressure Error FB I] and [HPP Pressure Error FB P] tables.

HPP Pressure is usually synchronously sampled and filtered.

HPP Cal and HPP Offset set the number and offset relative to engine synchronisation.

HPP Sample Weight n is used to reject noisy samples, more common at high flow rates.

HPP T Offset source selects the cam Timer input for correction.

To compensate for any errors in HPP Pressure, the flow versus pressure must be determined or estimated, using the [Injector Flow] table. To improve injector timing resolution Injection Range added, used to the set number of degrees that injection may start say for 200deg with 20 teeth/rev then 11 for 198crank degrees. There is a user scalar and offset to aid setup. If set to zero, Fuel Teeth is used as before.

Enable with HPP Injection Correct [HPP press inj cor]. This also enables the Injector Fuel Pressure Comp opening / closing time for HPP Pressure. This can help improve linearity of injector characteristic.

Sample Tables